COA

MINI PROJECT

By: G.Saiprasad 114A1012

Shweta Shekhar 116A1072

Sunil Pillai 116A1058

Aim: To implement 64 bit addition and subtraction using VHDL

Theory:

VHDL is a Hardware Description Language used for modelling digital systems made of interconnection of components. The complexity of the digital system being modelled may vary from that of simple gate to a complete electronic circuit/system. VHDL is an acronym for VHSIC Hardware Description Language and VHSIC is an acronym for Very High Speed Integrated Circuits. This language was first introduced in 1981 for the Department of Defence (DoD) under the VHSIC program.

### Describing a Design

In VHDL an entity is used to describe a hardware module. An entity can be described using,

- Entity declaration

- Architecture

- Configuration

- Package declaration

- Package body

### Entity Declaration

It defines the names, input output signals and modes of a hardware module.

Syntax −

An entity declaration should start with ‘entity’ and end with ‘end’ keywords. The direction will be input, output or inout.

| In | Port can be read |
| --- | --- |
| Out | Port can be written |
| Inout | Port can be read and written |
| Buffer | Port can be read and written, it can have only one source. |

Architecture −

Architecture can be described using structural, dataflow, behavioural or mixed style.

Syntax −

architecture architecture_name of entity_name

architecture_declarative_part;

begin

Statements;

end architecture_name;

Here, we should specify the entity name for which we are writing the architecture body. The architecture statements should be inside the ‘begin’ and ‘end’ keyword. Architecture declarative part may contain variables, constants, or component declaration.

In our program library IEEE is imported under which the package LOGIC_1164 & LOGIC_UNSIGNED is used.

An entity bitadd is created in which two ports of in type a, b and two ports of out type c, d are set. Input ports a & b are of Vector type each having 65 bits (64 downto 0).

Similarly, Output ports c & d are of Vector type automatically having 65 bits each.

An architecture add is created for the entity add which does the following operations:

- Addition (a + b) is carried out and result is stored in bit c.

- Subtraction (a – b) is carried out and result is stored in bit d.

Program:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity bitadd is

Port (

a : in  STD_LOGIC_VECTOR (64 downto 0) :=                   "00000000000000000000000000000000000000000000000000000000000000000";

b : in  STD_LOGIC_VECTOR (64 downto 0) := "00000000000000000000000000000000000000000000000000000000000000000";

c: out  STD_LOGIC_VECTOR (64 downto 0) ;

d: out  STD_LOGIC_VECTOR (64 downto 0)	);

end bitadd;

architecture add of bitadd is

begin

c <= a + b;

d <= a − b;

end add;

Example: To demonstrate (1234567898765432109)10 + (9876543210123456789)10

& (1234567898765432109)10 − (9876543210123456789)10

→

(1234567898765432109)10

= (1000100100010000100001111011010000001000000111001010100101101)2

(9876543210123456789)10

= (1000100100010000100001111011100010110000001101000111000100010101)2

(1000100100010000100001111011010000001000000111001010100101101)2

+ (1000100100010000100001111011100010110000001101000111000100010101)2

= (01001101000110010100110001010111100110001001110000000011001000010)2

= (11111111108888888898)10

(1000100100010000100001111011010000001000000111001010100101101)2

− (1000100100010000100001111011100010110000001101000111000100010101)2

= (11000100000010001100010010011110111010000110011110010010000011000)2

(In Signed 2’s Complement)

= (-8641975311358024680)

Output:

Input bit a:

Input bit b:

Output bit c (a+b):

Output bit d (a-b):